Water Quality Trends on the Penticton Oxbows and Adjacent Waters, 2019

Friends of the Oxbows (FOTO) have been measuring simple water quality parameters on the Penticton oxbows since 2015. We reported results in early 2017, and in 2018 showing trends for the total of our data base. Presented here is a further update of data we collected in 2019. As previously, data were collected using an Oakton PCTS Tester 50 for pH, conductivity, temperature, salinity, and total dissolved solids. We also collected data on turbidity, using a Triton Turbidity Wedge, converting those data into Nephelometric Trubidity Units (NTU) as reported previously. All previous reports can be found at www.pentictonoxbows.ca/publications.

pH

Acidity continued to fluctuate around pH 8, indicating a slightly basic nature to the water of both the oxbows and adjacent waters (Fig. 1). There was some variation in pH between months on individual water bodies, and no discernable trend over the course of the year (Fig. 2).

Conductivity

Conductivity was similar to that seen in previous years, except that no measurements were made during snow melt events at the Brandon Avenue oxbow. As a result there were no spikes in conductivity noted previously. The data for 2019 have been presented over the large range noted in previous reports to facilitate visual comparison with those earlier reports (Fig. 3). The overall average conductivity in 2019 was 446 µS/cm, compared to 437 µS/cm reported for data 2018 and older. There was a slight but not significant increase in average conductivity over the course of measurements in 2019 (Fig. 4).

Temperature

Temperature ranged from near freezing to nearly 30 °C (Fig. 5), similarly to data from previous years. Maximum temperatures were slightly lower than in previous years, consistent with trends in atmospheric temperature.

Salinity

Salinity remained below 0.5 pph throughout 2019 (Fig. 6), consistent with previous measurements. Water below that level is considered fresh, so the oxbow environment continues to exhibit low salt levels.

Total Dissolved Solids

Total dissolved solids, similar to salinity, remained low in 2019 (Fig. 7). The average for all waterbodies over the year was 340 mg/l, compared to 337 mg/l in 2018.

Turbidity

Turbidity again fluctuated throughout the year and between oxbows, while it remained very stable on the non-oxbow sites (Figs. 8 & 9). Average turbidity in the oxbows was 63 NTUs, while it remained at 40 NTUs in the non-oxbow water bodies. Once again, the water in oxbows was considerably more turbid than the surrounding waters, likely as a result of high sediments loads in the oxbows, and disturbance of those sediments by wind and the movement of large fish such as carp.

Conclusions

The oxbows and adjacent waters continued to exhibit characteristics of relative health in 2019, at least for pH, conductivity, salinity and total dissolved solids; differences between years for all parameters measured are not believed to be significant.  Water temperature in the oxbows, because they are of relatively small volume and have limited or no inflow, continued to fluctuate widely throughout the season in response to annual climate. Not unexpectedly, turbidity also remained higher than surrounding waters. Both of these factors (high temperatures during summer and high levels of turbidity) are likely to continue to limit general productivity of the oxbows, and for some species of fish are likely to be severely limiting.